# Report: Lab02_Routing

HUỲNh Ân

# 24560064 | 24560064@gm.uit.edu.vn

# EXPLAINATION of STATIC ROUTE, RIP, and OSPF

Static Route
1. Definition: Static Route is a method where you manually configure permanent routes fo the traffic to follow in the network.

- 2. Advantages: This method is very simple and easy to set up and it does not require system resources for calculating routes, but it is only suitable for small or stable networks.

- 3. Disadvantages: Static Route lacks flexibility and needs to update or change the configuration manually for network changes. Additionally, this method is not feasible for large and complex networks with multiple paths.

Rip
1. Definition: Rip is a dynamic routing protocol that uses a simple distance-based algorithm called “hop count”, to determine the best path for data.

2. Advantages: This protocol is easy to deploy and suitable for small networks and it can update the routing automatically between the routers.

3. Disadvantages: The Routing Information Protocol has a lack of hops, which limits 15 hops and thus is not suitable for big networks. Moreover, this protocol is slower and less efficient than modern routing protocols.

OSPF

- 1. Definition: OSPF is also dynamic routing protocol like RIP but more advanced and uses Dijkstra’s algorithm to calculate the shortest path between networks.

- 2. Advantages: This advanced method is feasible for large and complex networks, and it can split the network into regions for easier administration. Therefore, this protocol is very fast and efficient in spreading routing information.

- 3. Disadvantages: OSPF is more complex to configure than RIP and with the information propagation efficiency of OSPF, it requires more system resources.

# Process of Static Route

This stage starts by setting up static routes to connect the networks on the routers. In the Packet Tracer environment, open the Command Line Interface (CLI) tab to begin. After accessing the CLI on R1, the configuration is ready for adjusting network settings. Switch the router to configuration mode by entering the commands enable and configure terminal in order. On R1, type interface GigabitEthernet0/1 to access this interface, then type ip address 192.168.1.1 255.255.255.0 to assign the IP address, and type no shutdown to activate the interface. Next, type interface GigabitEthernet0/2, type ip address 192.168.12.1 255.255.255.0, and type no shutdown to enable the interface. Then, type ip route 192.168.2.0 255.255.255.0 192.168.12.2 to create a static route to the 192.168.2.0/24 network via R2. Repeat the process on R2, typing interface GigabitEthernet0/1, typing ip address 192.168.12.2 255.255.255.0, then no shutdown, followed by interface FastEthernet0/0, typing ip address 192.168.2.1 255.255.255.0, and no shutdown, then typing ip route 192.168.1.0 255.255.255.0 192.168.12.1 to create a static route to the 192.168.1.0/24 network via R1. In the final step, type the command show ip route on both R1 and R2 to verify the configuration, ensuring the setup works correctly, and you should see static routes marked with the symbol "S".

# Process of RIP

This stage starts by setting up the RIP protocol to allow routers to automatically share network information. In the Packet Tracer environment, open the Command Line Interface (CLI) tab to begin. After accessing the CLI on R1, the configuration is ready for adjusting network settings. Switch the router to configuration mode by entering the commands enable and configure terminal in order. On R1, type interface GigabitEthernet0/1 to access this interface, then type ip address 192.168.1.1 255.255.255.0 to assign the IP address, and type no shutdown to activate the interface. Next, type interface GigabitEthernet0/2, type ip address 192.168.12.1 255.255.255.0, and type no shutdown to enable the interface. Then, type router rip to start the RIP protocol, type version 2 to use RIP version 2 for subnet mask support, and declare the networks R1 needs to share by typing network 192.168.1.0 and network 192.168.12.0. Repeat the process on R2, typing interface GigabitEthernet0/1, typing ip address 192.168.12.2 255.255.255.0, then no shutdown, followed by interface FastEthernet0/0, typing ip address 192.168.2.1 255.255.255.0, and no shutdown, then typing router rip, followed by version 2, and declaring the networks with network 192.168.2.0 and network 192.168.12.0. In the final step, type the command show ip route on both R1 and R2 to verify the configuration, ensuring the setup works correctly, and you should see RIP routes marked with the symbol "R".

# Process of OSPF

This stage starts by setting up the OSPF protocol for more efficient routing. In the Packet Tracer environment, open the Command Line Interface (CLI) tab to begin. After accessing the CLI on R1, the configuration is ready for adjusting network settings. Switch the router to configuration mode by entering the commands enable and configure terminal in order. On R1, type interface GigabitEthernet0/1 to access this interface, then type ip address 192.168.1.1 255.255.255.0 to assign the IP address, and type no shutdown to activate the interface. Next, type interface GigabitEthernet0/2, type ip address 192.168.12.1 255.255.255.0, and type no shutdown to enable the interface. Then, type the command router ospf 1 to start the OSPF protocol with an ID of 1, and declare the networks to share by typing network 192.168.1.0 0.0.0.255 area 0 and network 192.168.12.0 0.0.0.255 area 0. Repeat the process on R2, typing interface GigabitEthernet0/1, typing ip address 192.168.12.2 255.255.255.0, then no shutdown, followed by interface FastEthernet0/0, typing ip address 192.168.2.1 255.255.255.0, and no shutdown, then typing router ospf 1, and declaring the networks with network 192.168.2.0 0.0.0.255 area 0 and network 192.168.12.0 0.0.0.255 area 0. In the final step, type the command show ip route on both R1 and R2 to verify the configuration, ensuring everything works correctly, and you should see OSPF routes marked with the symbol "O", then type show ip ospf neighbor to check if R1 and R2 are fully connected with each other.